# Harborline HP-3000 Heat Pump: Owner's Guide

Keep this guide with the unit. Model HP-3000, serial HL-000000-00.

# Filter Maintenance

Replace the 16 x 25 x 1 inch MERV 8 air filter every 90 days, or every 60 days when pets live in the house. A clogged filter reduces airflow and can freeze the coil.

## Checking the filter

Slide the filter out of the return grille in the hallway ceiling. Hold it up to a light; if you cannot see light through it, replace it.

# Thermostat Settings

The factory schedule uses these setpoints:

| Mode | Setpoint |
| --- | --- |
| Heat | 68 F |
| Cool | 76 F |

Hold the MODE button for five seconds to switch to emergency heat.

# Warranty Registration

The compressor carries a 10-year limited warranty when the unit is registered within 60 days of installation. Other parts carry a 5-year limited warranty. Registration number: HL-WR-000123.